[image: image1.wmf]
[image: image2.png]ACTIVE THERMAL
MANAGEMENT





Active Thermal Management

             The trusted name in thermal protection
Cool-vent II & III Installation Instructions 

The Cool-vents are compact self-contained air moving devices for use in enclosures housing heat-producing equipment. They can easily be field-modified to work in either intake or exhaust  mode. If used as an intake device, provisions must be made to allow the escape of hot air; similarly, when used in exhaust mode, there must be an opening through which  room air can enter. 

Cool-vents are powered by a plug-type power supply and controlled by a thermal switch. Cool-vent II and III are always on at a very low “idle” speed, and switch to a higher speed at 90 degrees. They mount on a finished surface through cutouts shown in the accompanying  chart. The fan assembly is easily separated from the wood grille to allow finishing the grille without damaging the fans.

	Model
	  Rough opening
	Finished size
	 Number/size of fans

	Cool-vent II
	4⅛" x 14⅛"
	5 ½" x 15 ½"
	4/80mm

	Cool-vent III
	6⅛" x 14⅛"
	      7 ⅛" x 15 ½"
	2/120mm


As shipped, Cool-vents are assembled as an intake device; air is pulled from the grille side through the fans. 

When the fan and grille sections are reattached, the Cool-vent can be assembled to operate in either intake or exhaust mode. 

· In intake mode, it can be mounted low on a cabinet to bring room air in. 

· In exhaust mode, it can be mounted above or behind an amplifier or another heat-producing component to expel heated air.

Main installation steps:

1. Separate the fan and grille sections.

2. Finish the grille as required.

3. Reassemble in intake or exhaust mode.

4. Install in opening.

5. Install thermal switch and connect to power source.

To install Cool-vent:

1. Separate the fan and grille sections by removing the nuts on the threaded studs. Use a 5/16" open-end wrench.  

2. Finish the wooden grille section as desired.  Allow to dry completely before reassembling to the fan section. (Read the following cautionary note.)

**IMPORTANT**

In order to protect the grille and prevent warping or cracking, you must seal the grille on EACH side and in between all slots prior to installation.
Whether you are painting, staining, or sealing the grille with urethane, they must have the appropriate finish applied BEFORE they are installed. If the grille is not protected in this way, Active Thermal Management will not be responsible for any warping or cracking that might occur.

3. Reassemble the two sections in desired operating mode. Do NOT use excessive force when tightening the nuts holding the fans.

a. For "intake" mode, reassemble as they were shipped, with the fans' labels facing away from the grille section. 

b. For "exhaust" mode, the fans' labels should face the grille section. 

4. Cut an opening of appropriate size (see chart above) at the desired location. Pass the thermal switch and power supply through the opening from the outside and screw or bolt the grille to the mounting surface either from the outside or inside as appropriate.

5. Attach the thermal switch to the equipment to be monitored, using one of the screws included, or mount the switch so that it can sense the temperature of the hottest air in the enclosure. (This might be at the highest point in the cabinet or just above the hottest component.)

To test the Cool-vent quickly, use a hair dryer to heat the thermal switch which will shift the fans to full speed operation.  
� EMBED AutoSketch.Drawing.6  ���





� EMBED Word.Picture.8  ���








25570 Rye Canyon  Rd.                                        Valencia, California  91355

(661) 294-7999  voice
                                                        (661) 294-1115 fax


techinfo@activethermal.com                                     www.activethermal.com

[image: image3.wmf][image: image4.png]ACTIVE THERMAL
MANAGEMENT




_1105530938.pcx

_1106335997.doc
[image: image1.png]ACTIVE THERMAL
MANAGEMENT








